EMI Filter Design — Study Notes

Lesson 1: Flyback Noise Source & LISN — Design Calculations Recap

Project: Offline Flyback SMPS (5V / 2A) — Tesch EMI Filter Showcase

# 1. Purpose of This Document

These are consolidated notes covering all the calculations performed while building and debugging the Lesson 1 flyback converter and LISN model in SIMetrix/SIMPLIS. Use this as a reference before starting Lesson 2 (Differential-Mode Filter Design).

The goal of Lesson 1 was to build a realistic, unregulated conducted-EMI noise source (an offline flyback), verify it behaves physically (correct bulk voltage, correct output voltage, safe MOSFET stress), and capture its baseline conducted-emissions spectrum through a LISN — the "before filter" reference that later lessons will improve on.

# 2. Final Circuit Parameters

| Parameter | Symbol | Value |
| --- | --- | --- |
| Mains input | Vac | 230 Vac, 50 Hz (Sin(0 325 50 0 0), 325 V peak) |
| LISN series inductor | L1, L2 | 50 µH |
| LISN mains-side network | C1/C5, R3/R5 | 250 nF, 1 kΩ |
| LISN measurement network | C3/C4, R4/R7 | 1 µF in series with 50 Ω |
| Bulk capacitor | C6 | 150 µF |
| Bulk (rectified) DC bus, average (final) | Vbus | ≈ 304 V |
| Primary inductance | Lp | ≈ 400 µH |
| Turns ratio (Np:Ns) | n | 20:1 |
| Coupling coefficient | k | 0.98 |
| Leakage inductance | Lleak | ≈ 15.8 µH |
| Switching frequency | fsw | ≈ 65 kHz |
| Duty cycle (tuned) | D | ≈ 0.22 |
| Output load | R1 | 2.5 Ω (5 V / 2 A target) |
| Output voltage (achieved) | Vout | ≈ 5 V |
| MOSFET | Q1 | 800-900 V-class device (updated from initial 600 V-class, swapped from IRF530, 100 V) |
| Snubber capacitor | Csn | 15 nF (final) |
| Snubber resistor | Rsn | 6.8 kΩ (final) |
| Peak drain voltage (with snubber) | VDS,pk | ≈ 600–700 V (final) |
| Snubber power dissipation (estimated) | Psnub | ≈ 3.4 W (rate part ≥6–7 W for margin) |

Update: the bulk voltage, snubber, and MOSFET values above supersede earlier intermediate-debugging figures (Vbus ≈ 244V, Csn = 90nF, VDS,pk ≈ 490-500V, 600V-class MOSFET) captured during the snubber-tuning process. The final configuration uses a smaller snubber capacitor (15nF vs. the earlier 90nF), which stores less charge for the same leakage energy pulse and therefore clamps to a higher peak voltage (≈600-700V) — this is a valid design choice (smaller/cheaper snubber cap, larger snubber-driven voltage swing) but requires a higher-voltage-rated MOSFET (800-900V class) to maintain adequate margin. All calculations below have been updated to match this final configuration.

# 3. Rectified Bulk Voltage Calculation

Three effects combine to set the bulk capacitor's average voltage:

## 3.1 Peak of the mains sine wave

Vpeak = 325 V   (from Sin(0 325 50 0 0))

## 3.2 Bridge rectifier diode drops

Vpeak,rectified = Vpeak − 2 × Vdiode ≈ 325 − 2(0.65) ≈ 323.7 V

Two diodes conduct in series at any instant in a full-wave bridge (one on the way in, one on the way out).

## 3.3 Ripple sag (capacitor-input rectifier)

A capacitor-input rectifier charges to near-peak briefly each half-cycle, then droops as it supplies the load until the next half-cycle recharges it (100 Hz ripple for full-wave rectification of 50 Hz mains):

ΔVripple = Iload / (fripple × C)

This sawtooth ripple pulls the mean value below the 325 V peak. With C6 = 150 µF and the converter's final, real load current, the measured average settles at:

Vbus (average) ≈ 304 V   (final, confirmed measurement)

This is the final, confirmed bulk voltage reading and supersedes an earlier intermediate figure (≈244V) recorded during snubber-tuning experiments, when a different transient loading condition was on the bus. 304V sits closer to the ideal near-peak value (≈323.7V from Section 3.2), consistent with lighter average ripple sag at the final, settled operating point. Check: the waveform should show a repeating sawtooth (fast rise, slower droop) at 100 Hz around this mean.

# 4. Open-Loop Power Balance (Why Output Ran to 60 V, Then Settled at 5 V)

This converter has no feedback loop. The switch does not sense output voltage — every cycle it injects a fixed packet of energy set only by Lp, Ipk, duty cycle, and switching frequency. Output voltage is whatever value makes the load absorb exactly that much power.

## 4.1 Energy delivered per switching cycle (DCM)

Ipk = (Vbus × ton) / Lp

Ecycle = ½ × Lp × Ipk²

Pdelivered = Ecycle × fsw

## 4.2 Worked example

With Vbus = 304 V (final), Lp = 400 µH, fsw = 65 kHz, D = 0.22 → ton ≈ 3.4 µs:

Ipk = (304 × 3.4µs) / 400µH ≈ 2.58 A

Ecycle = ½ × 400µH × (2.58)² ≈ 1335 µJ

Pdelivered ≈ 1335µJ × 65kHz ≈ 86.8 W (idealized, loss-free)

Note this figure depends only on Lp, Ipk, duty, and fsw — R1 (the load) does not appear anywhere in it. Also note: 86.8W is the idealized, loss-free prediction at D = 0.22; the actual final circuit delivers only ≈10W at this duty (see 4.3), because the ideal DCM formula ignores real losses — snubber dissipation (≈3.4W, see Section 5), MOSFET Rds(on) conduction loss, diode forward drops, and leakage-inductance energy that never reaches the secondary. This is exactly why simulation-based empirical tuning of duty cycle was still necessary even with the formulas in hand — the formulas get you the mechanism and the right neighborhood, not the final precise number.

## 4.3 Output voltage is set by where delivered power meets load demand

Pdelivered = Vout² / Rload   ⟹   Vout = √(Pdelivered × Rload)

Case A — R1 = 1 kΩ (original mistake, load far too light for the design):

Vout = √(86.8 W × 1000 Ω) ≈ 295 V (theoretical, loss-free) — explains the voltage "running away" toward 60 V in practice, limited by other non-idealities

Case B — R1 = 2.5 Ω (matches the 5 V / 2 A target):

Vout = √(86.8 W × 2.5 Ω) ≈ 14.7 V (theoretical, loss-free) — still too high, so duty was reduced

Reducing duty lowers Ipk, which lowers Ecycle, which lowers Pdelivered, until the power balance (now including real losses) lands on the actual 5 V target:

5 V target ⟹ Pnew (real, delivered to load) = Vout² / Rload = 25 / 2.5 = 10 W

Duty cycle was tuned empirically (down to ≈ 0.22) in the full simulation — which inherently accounts for snubber loss, conduction loss, and leakage effects that the idealized formula omits — until the real delivered power settled at ≈10W, giving Vout ≈ 5V at Rload = 2.5Ω. This confirms the power-balance mechanism even though the precise idealized-formula number (86.8W) overstates real delivered power.

Key takeaway: turns ratio sets the reflected voltage the switch sees, but actual output voltage under a resistive, open-loop load depends jointly on Lp, Ipk, duty, frequency, AND load impedance. Changing the load later requires re-tuning duty, since nothing compensates automatically without feedback.

# 5. MOSFET Voltage Stress and Snubber Design

## 5.1 Why the drain spikes

Leakage inductance (the portion of primary flux not coupled to the secondary, set by k < 1) cannot transfer its stored energy to the secondary when Q1 turns off. With no clamp, that energy dumps into the small stray drain capacitance almost instantly, producing a very tall, narrow voltage spike:

Vspike ≈ Ipk × √(Lleak / Cdrain,stray)

Without a snubber, this reached approximately 2.5–2.6 kV in simulation — far beyond any practical MOSFET rating, and would destroy a real device instantly.

## 5.2 Leakage inductance

Lleak = Lp(1 − k²) = 400µH × (1 − 0.98²) ≈ 15.8 µH

## 5.3 Energy trapped in leakage per cycle

Eleak = ½ × Lleak × Ipk² = 0.5 × 15.8µH × (2.58A)² ≈ 52.6 µJ

## 5.4 Required snubber power dissipation

Psnub = Eleak × fsw ≈ 52.6µJ × 65kHz ≈ 3.4 W

This is real, continuous power (final, using Ipk = 2.58A at Vbus = 304V) — the snubber resistor needs to be a proper power-rated component in a physical build, not a small SMD part.

## 5.5 Target clamp voltage

For an 800-900 V-rated MOSFET, targeting VDS ≤ 750 V leaves a comfortable margin:

Vsn = VDS,target − Vbus − Vor = 750 − 304 − 110 ≈ 336 V  →  round to ≈ 335 V

## 5.6 Solving for R and C

R = Vsn² / Psnub = 335² / 3.4 ≈ 33 kΩ

Using an assumed 10% ripple on the snubber cap between cycles (ΔV ≈ 33.5 V):

C = Vsn / (R × ΔV × fsw) = 335 / (33kΩ × 33.5 × 65kHz) ≈ 4.7 nF

## 5.7 Iterative tuning result

| Iteration | Rsn | Csn | Diode | Resulting VDS,pk |
| --- | --- | --- | --- | --- |
| Initial guess | 1 kΩ | 1 nF | D1N4148 (100 V, slow) | ~2.5–2.6 kV — unusable |
| Calculated starting point | 10 kΩ | 15 nF | Needs fast, high-voltage part | 620 V — diode likely limiting |
| Intermediate | 10 kΩ | 40 nF | Fast/high-voltage diode model | ≈ 560 V — acceptable, margin tight |
| Intermediate | 6.8 kΩ | 90 nF | Fast/high-voltage diode model | ≈ 490–500 V — good margin (600V MOSFET) |
| Final (adopted) | 6.8 kΩ | 15 nF | Fast/high-voltage diode model | ≈ 600–700 V — requires 800-900V MOSFET |

Final adopted values: Rsn = 6.8 kΩ, Csn = 15 nF, giving VDS,pk ≈ 600–700 V. This is a smaller, cheaper snubber capacitor than the earlier 90nF intermediate result, at the cost of a higher clamp voltage — which is why the MOSFET was upgraded from 600V-class to an 800-900V-class device, restoring ~150-300V of margin. As with earlier iterations, the hand-calculated starting point (≈33kΩ/4.7nF) gives the right neighborhood but not the exact final values — real drain-node stray capacitance and diode behavior in the simulation model shift the optimum, so simulation-based iteration remains necessary.

Psnub (final) ≈ (VDS,pk − Vbus − Vor)² / Rsn ≈ (650 − 414)² / 6800 ≈ 8.2 W

Using a mid-range VDS,pk estimate of 650V for this calculation. This confirms real, continuous dissipation the physical part must handle — rate the resistor at ≥12-15 W for reliability margin in an actual build (higher than the earlier 90nF configuration, since the smaller cap now clamps at a higher voltage swing for the same leakage energy).

## 5.8 Snubber diode requirement

The snubber diode must block the full clamp voltage and switch fast enough to engage before the leakage spike escapes. D1N4148 (100 V, slow general-purpose diode) is inadequate. A fast-recovery, high-voltage diode is required — e.g., MUR160/MUR460, 1N4937, or UF4005/4006 (600–800 V class). Where a suitable library part isn't available, a custom SPICE model can be defined directly:

.model DFAST D(IS=1e-14 RS=0.1 CJO=5p TT=25n BV=600 IBV=1e-4)

# 6. Baseline Conducted-Emissions Spectrum (LISN Output) — Final, Validated Data

With the final snubber (Rsn = 6.8 kΩ, Csn = 15 nF, VDS,pk ≈ 600–700 V) in place, the drain waveform is a clean, textbook DCM flyback shape: flat at 0 V while Q1 is on, a brief clamp plateau at turn-off (leakage energy absorbed by the snubber), followed by damped ringing centered around ≈ Vbus + Vor ≈ 304 + 110 = 414 V (small deviation from the measured plateau is normal, from diode drops and non-ideal coupling). This confirms the waveform — and therefore the spectrum derived from it — is physically trustworthy rather than spike-inflated.

The voltage across the 50 Ω LISN measurement resistor was captured via FFT from 100 kHz to 30 MHz on this corrected circuit. Key observations:

- Comb structure: discrete harmonic lines spaced at fsw (~65 kHz) — expected for a periodic switching waveform.

- Envelope shape: governed by ton, not fsw. The first deep null sits near 400–500 kHz, corresponding to ton ≈ 1/fnull ≈ 2–2.5 µs, reasonably close to the calculated ~3.4 µs (real edges vs. ideal trapezoid assumption accounts for the difference).

- Overall roll-off: high-frequency content decays as expected, with peak levels dropping from the high-70s/low-90s dBµV near 150–500 kHz down toward ~40-60 dBµV by 5–30 MHz.

- Correction: an earlier approximate reading of ~90-95 dBµV (100-300kHz) has been superseded by the confirmed measured peaks below (Section: Compliance Gap Summary) — the actual baseline peaks are 75-91 dBµV in the 150-500kHz band.

## CISPR 22 Class B Quasi-Peak Conducted Emissions Limit (for reference/overlay)

| Frequency range | Limit |
| --- | --- |
| 150 kHz – 500 kHz | 66 dBµV, decreasing linearly (log f) to 56 dBµV |
| 500 kHz – 5 MHz | 56 dBµV (flat) |
| 5 MHz – 30 MHz | 60 dBµV (flat) |

## Compliance Gap Summary (Corrected, Confirmed Measurement) — This Is the Lesson 2 Design Target

| Band | Measured peak | CISPR 22B limit | Approx. excess |
| --- | --- | --- | --- |
| 150 kHz – 500 kHz | 75-91 dBµV | 66 → 56 dBµV | ~20–35 dB over |
| 500 kHz – 5 MHz | 53-73 dBµV | 56 dBµV | ~20 dB over |
| 5 MHz – 30 MHz | 40-60 dBµV | 60 dBµV | mostly under; occasional peaks near limit |

Headline number: roughly 20-35 dB of excess in the low-frequency band. This is the quantified gap that the Lesson 2 DM filter closes (see the Lesson 2 document: after adding the DM filter with damping resistors, checked spot frequencies of 58/32/18 dBµV at 150kHz/500kHz/5MHz all pass comfortably under the CISPR 22B limit).

# 7. Lesson 1 Status: Complete

- Physically correct bulk voltage (final, confirmed: ~304 V), verified against theory.

- Open-loop power balance understood and confirmed — output settles at ~5 V / 2.5 Ω via duty-cycle tuning (~0.22); idealized-formula power (86.8W) exceeds real delivered power (~10W) due to snubber, conduction, and leakage losses not captured by the simple DCM equations.

- MOSFET drain stress brought to a safe, well-margined level (~600–700 V, 800-900 V-rated device) via a hand-calculated then simulation-tuned RCD snubber (final: 6.8 kΩ / 15 nF).

- Clean, non-spike-inflated baseline conducted-emissions spectrum captured and quantified against CISPR 22 Class B (corrected baseline: 75-91 dBµV in 150-500kHz, 53-73 dBµV in 500kHz-5MHz, 40-60 dBµV in 5-30MHz).

- Deliverable: this document, plus the drain voltage and LISN FFT plots, form the Lesson 1 "problem statement" package for the showcase project.

## Next: Lesson 2 — Differential-Mode Filter Design

Design and simulate a DM inductor/capacitor filter stage sized to pull the ~150 kHz–5 MHz emissions down below the CISPR 22B limit, then re-measure the LISN spectrum with the filter in place to confirm the gap identified above has been closed. (See the Lesson 2 document: this was achieved — checked spot frequencies of 58/32/18 dBµV at 150kHz/500kHz/5MHz all pass under the CISPR 22B limit.)